All of the HIMA products mentioned in this manual are trademark protected. Unless noted otherwise, this also applies to other manufacturers and their respective products referred herein.

HIQuad®, HIQuad® X, HIMax®, HIMatrix®, SILworX®, XMR® and HICore® are registered trademarks of HIMA Paul Hildebrandt GmbH.

All of the technical specifications and information in this manual were prepared with great care and effective control measures were employed for their compilation. For questions, please contact HIMA directly. HIMA appreciates any suggestion on which information should be included in the manual.

Equipment subject to change without notice. HIMA also reserves the right to modify the written material without prior notice.

Registered customers can download the product documentation from the HIMA Extranet.

© Copyright 2024, HIMA Paul Hildebrandt GmbH

All rights reserved.

Contact

HIMA Paul Hildebrandt GmbH

P.O. Box 1261

68777 Brühl, Germany

Tel.: +49 6202 709-0

Fax: +49 6202 709-107

E-Mail: info@hima.com

| Document designation | Description |
| --- | --- |
| SILworX Plugin Asset_Inventory D Rev. 1.3 (2428) | German original document |
| SILworX Plugin Asset_Inventory E Rev. 1.1 (2428) | English translation of the German original document |

Table of Contents

1	SILworX Plug-Ins (Option Subject to License)	5

2	Configuring Plug-Ins	6

2.1	Installing Plug-Ins Using the Plug-In Manager	6

2.2	Manual Configuration of the Plug-In	6

2.2.1	Creating the SSL Certificate and Key	7

2.2.2	Completing the «settings.ini»	7

2.2.3	Plug-In-Registration	8

2.2.4	Starting Plug-Ins	9

2.2.5	Completing the «plugins.json»	9

3	Functionality of the Plug-In «Asset_Inventory»	11

SILworX Plug-Ins (Option Subject to License)

SILworX can be equipped with plug-ins to enhance its functionality. Plug-ins are non-safety-related applications that interact with SILworX, but are not contained in the SILworX development environment and are not certified along with SILworX.

Plug-ins enable users to create additional functions according to their own needs or to make these available to others. The prerequisite is that the necessary licenses exist. Plug-ins created by the customer can be used with the Plug-In Feature license in combination with the SILworX API license. Use of the license-free plug-ins provided by HIMA requires only the SILworX license.

Plug-ins run as independent processes in a memory area separate from SILworX and interact with SILworX via the SILworX API and WebSocket interfaces using encrypted connections. Plug-ins can be integrated interference-free and do not impair the safe SILworX functions in any way.

Because plug-ins are not an integral component of SILworX, they must be configured in the settings.ini file. To this end, the settings.ini file includes the [Plugin_Server] category. When started, SILworX checks whether the registered plug-ins exist and readies them so the functionality they offer is available in SILworX. SILworX starts the plug-ins listed in settings.ini under PluginNames automatically. The sequence of the plug-in names determines the processing priority of the plug-ins in the event of simultaneous triggering.

Plug-ins evaluate predefined triggers provided by SILworX as well as user-specific triggers and execute the functions required by the user. The actions triggered in SILworX by plug-ins are logged in the logbook. A separate area to the right of the API information in the Status Bar displays information about the plug-in feature.

The plug-ins that are started have no separate user management. Authentication takes place when the project is opened in SILworX. The user session this creates is also available to the plug-ins and the SILworX API.

When the plug-in feature has been started, you can use the Plug-In Information menu function in the SILworX Help menu to open a dialog box. The dialog box contains the two lists Configured Plug-Ins and Registered Plug-Ins. The dialog box shows the actions with which a plug-in in SILworX is linked and which triggers from SILworX or to SILworX are registered.

Summary:

Plug-ins are SILworX add-ons that can be created by users themselves.

Plug-ins are based on programs or scripts, possibly with additional files, that are configured in SILworX.

Plug-ins are independent modules that run in separate processes.

Plug-ins are started automatically along with SILworX.

Plug-ins have access to SILworX via the SILworX API functions.

Plug-ins require a Plug-In Feature license as well as a SILworX API license.

Plug-ins can react to predefined triggers as well as user-defined triggers.

The execution of plug-ins is displayed in the SILworX status bar.

Configuring Plug-Ins

If you wish to enhance SILworX with additional functions using plug-ins, you must configure the required plug-ins correctly in SILworX. The processes this entails require numerous steps.

You can either carry out these steps completely manually or let Plug-In Manager assist you. The Plug-In Manager itself is a license-free plug-in provided by HIMA that you may use free of charge. If you select the options Create a TLS Certificate for SILworX API and Plug Ins and Activate HIMA Plug Ins when installing SILworX, then the additional menu functions Install Plug Ins and Uninstall Plug Ins are available in the SILworX Extras menu

Installing Plug-Ins Using the Plug-In Manager

To install the plug-in or to update an existing plug-in, open the Installing Plug-Ins menu item in the Extras menu. A dialog box will prompt you to select a plug-in archive with the extension «.silworx_plugin». The metadata from the plug-in archive are displayed to identify the selected plug-in. When the Install button is confirmed, the plug-in is added to SILworX.

If SILworX already contains a plug-in whose name is identical with the name from the selected plug-in archive, the meta information with the differences is displayed. When the Update button is clicked, the existing plug-in is replaced by the new plug-in. The replacement takes place even if there are differences in the meta information.

Manual Configuration of the Plug-In

To use SILworX plug-ins requires that you save the desired plug-ins to a drive that is accessible to SILworX and configure them in the settings.ini file. You also require a Plug-In Feature license, a SILworX API license and a SILworX API certificate. Access to the SILworX API is only possible via SSL (TLS 1.2 or TLS 1.3).

The use of SSL requires a valid certificate and the associated private key. The paths to these files must be entered in the settings.ini file in the category [API_Server]. Using plug-ins in combination with the SILworX API requires that the configuration parameter Enabled of the category [API_Server] in the settings.ini is set to TRUE.

When starting, SILworX imports various plug-in configuration parameters from the settings.ini file, which are grouped in the category [Plugin_Server]. After this, SILworX checks the integrity of the registered plug-ins based on the hash codes. If the result of the integrity check is positive, SILworX makes the plug-in functions available.

The names of the plug-ins to be used in SILworX must be present in the PlugInNames list separated by commas. If several plug-ins respond to a certain trigger, their sequence in the PlugInNames list determines the order in which the plug-ins are executed.

A maximum of 254 plug-ins can be registered.

If PlugInNames contains plug-in names that occur more than once, the duplicates are rejected during the import of the plug-in configuration and SILworX reports an error.

The plug-in names are defined in the metadata of the plug-ins. They can differ from the names of the files used for the plug-ins.

The plug-ins listed in PluginNames must be executable programs (*.exe) and, to protect against unauthorized tampering, each of these plug-ins must be configured in the hash file plugins.json.

SILworX installs several HIMA plug-ins and enters their names as PluginNames by default. You can supplement this list at any location with your own PluginNames. The API server must also be Enabled to allow use of the HIMA plug-ins.

After initialization, SILworX automatically starts all of the plug-ins configured in PluginNames. Doing so requires that the appropriate hash codes for the plug-ins listed in PluginNames are present in plugins.json. The data paths are also taken from plugins.json. SILworX issues messages to the logbook if plug-ins cannot be started.

Creating the SSL Certificate and Key

To use the SILworX API, you need an SSL certificate in the PEM format, which you must create yourself. In addition to the certificate, you also need the corresponding private key in the PEM format that was created with the RSA algorithm without a passphrase.

The certificate and key must be entered in the SILworX settings.ini file.

Observe the period of validity of the SSL certificate!

When you create a new SSL certificate, you can define its period of validity in days. HIMA recommends limiting the period of validity to 365 days or less. Some clients (e.g., current browsers) do not accept any self-signed certificates with a validity period of greater than 365 days.

To create a valid certificate

Select in SILworX Extras, Create SILworX-API certificate from the main menu. The Create SILworX-API certificate dialog box appears.

Enter a path for the destination directory and the certificate name in the Filepath - Certificate field or use the button on the right-hand side, next to the field, to specify the path and name in a dialog box.

Enter a path for the destination directory and the key name in the Filepath – Key field or use the button on the right-hand side, next to the field, to specify the path and name in a dialog box.

If required, fill in the optional fields with your data.

Click Create to confirm the action.

Completing the «settings.ini»

To configure the SILworX API and the plug-ins, you must complete the following entries to the settings.ini file.

Open the settings.ini file in the SILworX progam data directory %ProgramData%\SILworX_vXX.X.0 Rxxxx\settings\, for example C:\ProgramData\SILworX_v15.0.0 R2875\settings\settings.ini, with a text editor.

Search in the settings.ini file for the entry [API_Server]. If this entry does not exist, append it to the end of the file.

The following entries are required under [API_Server], and you must complete them as needed.

	Enabled=true
	ServerCertificate=Absolute path to the certificate file
	ServerPrivateKey=Absolute path to the key file

Search in the settings.ini file for the entry [Plugin_Server]. If this entry does not exist, append it to the end of the file.

The following entries are required under [Plugin_Server], and you must complete them as needed.

	Enabled=true
	PluginNames=List of plug-ins names separated by commas

Example for the «settings.ini»

...
[API_Server]
Enabled=true
ServerCertificate=C:\Plugins\Certificates\api_cert.pem
ServerPrivateKey=C:\Plugins\Certificates\api_key.pem
[Plugin_Server]
Enabled=true
PluginNames=hima.asset_inventory

Plug-In-Registration

In the registration that follows a successful WebSocket connection set-up, the plug-ins log in to SILworX with their names, the metadata and their required triggers. This requires that the plug-ins be correctly configured with their names in settings.ini.

Plug-ins log in after they are started by means of messages, which they send to SILworX. If the registration was successful, messages with the file path of the plug-ins are written to the logbook. Also the hash codes of the plug-ins are shown.

Avoiding Errors During Plug-In Registration

If the registration was not successful, an error message is displayed in the logbook with an indication of the cause of the failed registration. To avoid errors, observe the following notes:

Multiple registration of plug-ins
No duplicate names may occur in the configuration parameter PluginNames of settings.ini.

Inadmissible registration of a plug-in
The plugin name defined in the metadata of a plug-in must be listed in the configuration parameter PluginNames of settings.ini. This plugin name must also be assigned as a value to the plugin_name key in plugins.json.

Maximum number of plug-ins
In SILworX, you can register a maximum of 254 plug-ins simultaneously.

Maximum number of triggers
The maximum number of 20 trigger entries for the Extras menu and 100 trigger entries for the total of all context menus must not be exceeded.

Incorrect formatting of hash file
Pay attention to the correct number of opening and closing brackets and double quotation marks. Whitespaces outside of string values are not significant. The path to the plug-in file must contain slashes / or double backslashes \\, single backslashes \ are not permitted.

Starting Plug-Ins

Starting plug-ins means that plug-ins are first registered in SILworX so they can interact with SILworX. After successful plug-in registration, the programs are present in the working memory of the computer. The actual interactions with SILworX and data processing are triggered by trigger notifications that SILworX sends to the plug-ins.

Upon initialization, SILworX starts a plug-in process for every plug-in that is configured in settings.ini under PluginNames, provided the plug-in data are entered in the plug-in hash file. If these data do not exist, SILworX issues a message and does not start the plug-in.

SILworX starts the plug-ins listed in the settings.ini automatically. When a plug-in is started, SILworX passes a command line with some parameters and generates a message with the content of the command line in the logbook.

The plug-ins that are started have no separate user management. Authentication takes place when the project is opened in SILworX. The user session this creates is also available to the plug-ins and the SILworX API.

Completing the «plugins.json»

The authenticity of the plug-ins configured in settings.ini under PluginNames is checked by SILworX before they are registered. The check is made by comparing the currently calculated hash codes of the plug-ins with the nominal values provided by the producer of the plug-ins that were previously stored in a hash file.

When importing the plug-in configuration and hash file, SILworX discards those plug-in names that occur twice and reports them as errors.

The hash file plugins.json contains the hash file codes of the plug-ins and must be adapted by the customer as needed. The file contains a separate data block for each plug-in and is structured as follows:

{
  "plugins": [
    {
      "plugin_hash": "Hash code according to SHA-256 plug-in 1",
      "plugin_name": "Plug-in name according to the meta data of plug-in 1",
      "plugin_path": "Absolute path to plug-in 1"
    },
    {
      "plugin_hash": "Hash code according to SHA-256 plug-in 2",
      "plugin_name": "Plug-in name according to the meta data of plug-in 2",
      "plugin_path": "Absolute path to plug-in 2"
    }
  ]
}

To configure the plug-ins, you must complete the following entries to the plugins.json file.

Open the plugins.json file in the SILworX progam data directory %ProgramData%\SILworX_vXX.X.0 Rxxxx\plugins\, for example C:\ProgramData\SILworX_v15.0.0 R2875\plugins\plugins.json, with a text editor.

The hash file contains a separate data block for each plug-in, consisting of a list of key : value pairs enclosed in curly brackets:

{
	“plugin_hash“ : “Hash code according to SHA-256“,
	“plugin_name“ : “Plug-in name according to the meta data of the plug-in“,
	“plugin_path“ : “Absolute path to the plug-in“
}

Add the appropriate values for the plug-in to this data block in the plugins.json file. If this data block does not exist, insert it after the opening square bracket. Please note that the string values must be enclosed in double straight quotation marks ".

Backslashs \ in JSON strings

In accordance with the JSON format definition, the backslash \ is used in JSON strings as a masking character to introduce escape sequences. Therefore, the backslashes in path specifications must either be replaced by double backslashes \\ or by slashes /.

Example for the «plugins.json»

{
  "plugins": [
    {
      "plugin_hash": 
           "AE682A36F20A5A5667149F0F5F317AE11C2DB39420C28E93003A829CBEC8B468",
      "plugin_name": "hima.asset_inventory",
      "plugin_path": "C:/Plugins/Asset_Inventory.exe"
    }
  ]
}

Functionality of the Plug-In «Asset_Inventory»

The plug-in is started via the context menu of a resource:

Figure 1: Resource context menu with menu entry for the plug-in

The plug-in retrieves data from SILworX, the open project and the running controller and then saves the collected data in a CSV file for further processing.

To do this, it executes the following SILworX actions one after the other:

Retrieving of product name, version and license via the Help, About SILworX menu

Retrieving of the project name from the structure tree

Retrieving of the system ID of the selected resource via the Extras, Properties menu

Execution of a system login with access mode "Read" via the Extras, Online menu of the selected resource

Retrieving of the main system and status data of the connected resource from the control panel

Retrieving of the hardware module information of the connected resource via the Online, Module Data Overview menu

Disconnecting the online session of the connected resource via the Online, Disconnect menu

During data recording, the plug-in displays a message indicating that the actions are being carried out.

Figure 2: Feedback message of the plug-in

If the SILworX project has a user management, the plug-in cannot perform a system login with the default login credentials. In this case, the plug-in requests the login data for the controller via a dialog.

Figure 3: Login dialog of the plug-in for the system login

Once all inventory data has been retrieved, another dialog box appears to save the data in CSV format.

Figure 4: Dialog box of the plug-in for saving the inventory data

By default, the CSV file is stored in the user directory %UserProfile%\Documents\ under the name "Inventory_yyyy_mm_dd_hh_mm_ss.csv", where "yyyy_mm_dd_hh_mm_ss" corresponds to the date and time the plug-in was started. The CSV file contains the information shown below as an example in tabular form, which can be further processed with any spreadsheet program or database system.

Figure 5: Section of an example with the inventory data from the CSV file

Additionally, the plug-in creates a log file with the name «hima.asset_inventory.log» in the %AppData%\SILworX_Plugins\ directory. The log file contains all status and error messages that have occurred and the time at which the message was issued. The output is in tabular form.

# Appendix

# Glossary

| Term | Description |
| --- | --- |
| SILworX | Programming tool for HIMax, HIQuad X and HIMatrix |
| API | Application Programming Interface |
| HTTPS | HyperText Transfer Protocol Secure, communication protocol |
| SSL | Secure Sockets Layer, deprecated ancestor of TLS |
| TLS | Transport Layer Security, cryptographic protocol |
| PEM | Privacy Enhanced Mail, file format according RFC 7468 |
| RSA | Rivest–Shamir–Adleman, public-key cryptosystem |
| WebSocket | TCP based communications protocol according RFC 6455 |
| SHA | Secure Hash Algorithm, cryptographic hash function |
| JSON | JavaScript Object Notation, file format according RFC 8259 |
| CSV | Comma-Separated Values, file format according RFC 4180 |

Index of Figures

Figure 1:	Resource context menu with menu entry for the plug-in	11

Figure 2:	Feedback message of the plug-in	12

Figure 3:	Login dialog of the plug-in for the system login	12

Figure 4:	Dialog box of the plug-in for saving the inventory data	13

Figure 5:	Section of an example with the inventory data from the CSV file	13